Portable drilling machine

These safety guidelines should be read in conjunction with the Portable hand-held power tools – general notes.

### Specific hazards associated with their use:

- loss of control due to torque generated during machine start-up, or if a drill bit jams

- the chuck key being violently ejected from the chuck on start up

- the drill point slipping from desired position at the start of the drilling operation

- torque generated if student stops and re-starts drill, with the drill bit embedded in the workpiece

- the drill breaking in use

- sudden movement as drill bit cuts through material

- skin contact with hot drill bit after use

- contact with the sharp burr generated when drilling metals

- drill bit hitting hidden electrical cable or other hidden  services when drilling into walls or floors.

### Control measures

- Once a student has demonstrated competency and control with this tool, close teacher supervision should be exercised.

- Use a battery powered, cordless machine if possible.

- A key-less chuck should be used whenever possible – if a chuck key is used, it must be stored away from the chuck before the drill is switched on (for masonry work, an SDS drilling machine and bits are better).

- Check drill size, speed and material to ensure suitable match between strength and maturity of student and operating characteristics of machine (resulting torque and feed rates required).

- Use pilot holes to drill holes stage by stage if necessary.

- Be aware of heat build up and potentially sharp swarf being produced if drilling metals.

- Make sure the location to be drilled is pre-marked, indented, or taped to minimise the chances of the drill wandering when cutting starts.

- Machine operators must de-burr metal surfaces immediately following drilling operations.

- Students must always wear goggles or safety glasses. When necessary, students must wear ear defenders and a dust mask. Long hair, jewellery and any loose clothing