Portable nail gun

These safety guidelines should be read in conjunction with the Portable hand held power tools – general notes.

### Specific hazards associated with their use:

- the nail gun is a popular nailing device used in light and heavy construction. It uses the force from compressed air or an electric solenoid to drive nails through hard surfaces

- compressed air powered nail guns are essentially a fire arm and must be treated accordingly

- injury to eyes and face caused by unexpected break up of timber as nails are propelled from gun

- injury to body parts caused by kick-back of nail gun or nails (if nail is fired against very hard materials)

- penetration of body of user or other person by propelled nail in the vicinity of the nail gun operation.

### Control measures

- The nail gun must only be operated by competent adults who have been trained it the safe use of the equipment.

- All elements of the manufacturers safety guidelines must be adhered to.

- Compressed air powered nail guns must not be operated in any area where students are present.

- Nails guns must not be operated by students under the age of 18.

- All adults in the vicinity of nail gun operations must wear eye protection.

- Care must be taken to ensure that people are not in line with nail gun operations due to the risk of nails being fired through timber sections.

- The trigger should only be held in or pressed when there is an intention to fire a nail – the trigger must not be held on to produce repeat nailing actions.

- The nail gun must only be fired when it is held against a work surface.

- Only use nails as specified by the manufacturer.

- Nail guns must not be used by operators on ladders.

- Nail guns must never be pointed in the direction of people or open space and must be carried facing down.

- Operators must not place their hands behind the workpiece to support the work – the nail may penetrate through the workpiece.

- Nails must not be re-used in the nail gun.

- Nails may deviate whilst passing into timber due to knots, etc. Operators should be aware of this and plan the nail operation accordingly.

- Compressed air nail guns must be regularly serviced by authorised and reputable specialist dealers.