Design and technology 
health and safety guidelines

## Risk assessment – cross-cutting/
mitre saw

### The principle hazards arise from:

- the circular cutting blade rotating at high speed, and the risk of severe injury likely to be caused by anyone coming into contact with the blade

- cuts to hands and fingers caused by handling the blade during maintenance or blade changing operations

- the risk of damage to the eyes caused by dust or chippings

- inhalation of wood dust

- damage to ears caused by noise

- the operator being distracted whilst using the machine

- any inadvertent or unauthorised starting up of the machine

- the machine operator tripping or stumbling whilst using the machine caused by debris, clutter material offcuts or electrical cables on the floor.

### Control measures – machine installation issues

- Cross cut mitre saws to be used in a school must be of suitable construction, and must comply with the requirements contained in BS 4163: 2014 and the Provision and Use of Work Equipment Regulations (PUWER) Approved Code of Practice (ACOP).

- The machine must be securely fixed in place, and mounted so that it is level and stable.

- Saws must be used indoors and not exposed to rain or dampness.

- A separate electrical isolator must be used to connect the machine to the electrical supply.

- The electrical isolator or plug must be clearly labelled.

- The machine must be fitted with a 0 volt (0v) starter.

- A key operated electrical switch is required to prevent unauthorised use.

- All fixed guards and covers must be fitted with either a microswitch interlock or a tool operated fixing.

- A foot or other conveniently operated stop switch must be fitted to enable the operator to switch off the machine without letting go of the controls or workpiece if necessary.

- Machines must be fitted with a spring assisted return mechanism so the blade returns to a safe, rest position when released.

- There must be adequate lighting, enabling the machine operator to have good visibility of all parts of the machine, free from any flickering lights which may cause a stroboscopic effect.

- The machine and controls must be positioned so the operator can maintain balance and does not have to over reach.

- The workpiece must be adequately supported along its length, so it does not tip, become unbalanced and be able to be slid into position within the machine. Therefore sufficient space must be provided at either side of the saw, and a means of supporting long lengths of material to be cut.

- Clamps should be provided to enable clamping of the 
workpiece in place whenever possible.

- It is good practice to have a no-hands area marked on the cutting table extending 150mm at either side of the blade.

- Effective dust extraction must be provided to comply with the Control of Substances Hazardous to Health (COSHH) regulations.

- The blade must stop rotating within 10 seconds of the machine being switched off.

- The floor area must be kept clear and be non-slip.

- A notice must be fixed to the machine, or be in the vicinity of the machine, listing the authorised users of the machine, date of their most recent training, and reminding users to wear eye protection.

- The machine must be included in a planned maintenance programme to include COSHH testing for the dust extraction system, and electrical safety testing.

### Control measures – machine operation and supervision issues

- Only trained adults, holding an approved certificate of competence should be allowed to use this machine.

- Properly trained, but inexperienced users must be supervised until they have demonstrated sufficient competency and confidence in the use of the machine.

- The blade must be regularly checked for damage, secure fixing and wear/bluntness whenever it is in use.

- Pupils must not be in the room whilst the saw is in operation.

- The blade teeth must be regularly inspected for wear or damage. Damaged blades must be replaced.

- All machine surfaces must be kept clear of tools and
offcuts when in use.

- Reclaimed materials must be closely inspected for nails, screws, wire or any other foreign bodies before being cut on the saw. Any materials from construction sites should be wire brushed before being cut on the saw.

- Operators must not be distracted whilst using this machine.

- Operators must not try to force the cutting rate on the machine, but work within the machine capacity.

- The nature of the required cutting operation and the length, width and thickness of the material to be cut must be within the capacities of the machine and the operator.

- Only materials recommended by the machine manufacturer should be cut using the machine.

- Check that all guards must be in place, and are fully functional before cutting materials.

- Make sure all adjustments and clamps are secure before starting the machine, eg: any mitre cutting or bevel cutting settings.

- Always allow the machine to reach its maximum speed before starting cutting operations.

- When short pieces of material are required to be cut, and the operators hands would be inside the 150mm exclusion zone, clamps or jigs should be used to secure the workpiece.

- Never place either hand in the blade cutting area when the machine is plugged in or connected to the power supply.

- Never attempt to stop or slow the blade down after it has been switched off.

- At the end of a cutting operation, always keep both hands in position until the blade has come to a standstill.

- When setting up for a cutting operation, always perform a dry run without the power switched on to make sure the cutting operation can be completed.

- Always ensure the blade is securely mounted and fixed in place before use.

- Always use a suitable blade to match the material to be cut.

- Only use saw blades sizes that are within the range specified in the machine operators manual.

- The blade guard should never be raised manually unless the machine is unplugged or electrically isolated.

- Local exhaust ventilation (LEV) must be switched on when the machine is in use.

- Goggles must be worn.

- Consider wearing ear protectors if using the saw for an extended period of time, or if noise levels are uncomfortable.

- When using the machine, loose clothing and jewellery must not be worn, non-slip shoes should be worn, and long hair tied back.

- The area around the saw must be clear, and the floor must be non-slip.

- The key, which operates the electrical switch, must be removed when the user has completed the cutting operation.

- The machine must not be left unattended when the key is in place.

- The machine must be electrically isolated whilst any adjustments are made.

- Any tools or other equipment used to set the machine up or make adjustments should be conveniently stored on a shadow board near to the machine.

- In all respects, the use of the saw must comply with the Safe use of woodworking machinery: provision and use of work equipment regulations1998( as applied to woodworking machinery) – Approved Code of Practice and guidance, by Health and Safety Executive (HSE), ISBN: 978 0 7176 6621 8

### Control measures – maintenance of 
cross-cutting/mitre saws

- A maintenance log must be kept for each machine.

- All slide ways and pivots and linkages should be kept suitably lubricated and free running.

- Repairs to the machine must be undertaken by authorised and qualified technicians.

- Check machine is securely fixed in position and all electrical connections, including conduits, are soundly made.

- The blade must be maintained in a sharp condition.

- Handles and switches must be kept clean.

- Check and maintain the electrical lead, switches and plug, these must be in an undamaged condition.

- Check and ensure safe and effective operation of all electrical systems including:

- direct online 0v starter

- key operated switch

- foot stop switch

- any interlocked machine guards or cabinet plates.

- Check and ensure machine run-down time is less than 10 seconds.

- Check with most recent COSHH report for any remedial work needed to ensure effective dust extraction.

- Check condition of blade and report any damage to an appropriate member of staff.